Multi-function Reader Device User Manual

LIF series multi-function reader (model: MSR006) is a USB interface multi-function device with magnetic Card reading, iButton reading, contact IC Card reading and writing, RFID Card reading and writing.

- Overview：

LIF multi-function reader, with magnetic card reading, iButton reading, contact IC card reading and writing, RFID card reading and other functions.

The card head bracket length is 90mm, black.

Multi-function Reader

Description：Using MAGTEK decoding IC,decoding board model:LIF-MR3。

PCB Model No.：Motherboard：Vn1.1;

Decoding board model：Vn1.1；

Software Model No.：Vn1.0

- B: Physical Characteristics:

● Main material: ABS, black;

● Size (L*W*H) : support: 90*21*21.5mm;

● Net weight of equipment: 75g (excluding cables and other accessories);

card slot stand appearance and structure

- Schematic diagram of functional structure composition

PC

（USB port）

Multi-function Reader

iButton Reader

MSR+main touch panel

RFID card reader

Contacted IC card stand

USB OTG port

C: Electrical characteristics:

- voltage: DC+5V ±10%;

- current: ≤150mA;

- Low insulation resistance: > 50 m Ω;

- Low contact resistance: < 1 k Ω.

- D: Magnetic card reader features:

- In line with ISO7811 standard, compatible with IBM,ANSI,JS2 and other protocols and standards;

- ● card swiping speed: 10-120cm/s;

- ● magnetic head life ≥500000 times;

- ● support two-way card slot;

- ● magnetic stripe coercivity range: 300-4000 Oe;

- ● magnetic stripe density: 210BPI for track 1; Track 2, 75/210BPI; Track 3 210BPI;

- ● available configuration: 1 track or 2 track or 3 track; Track 1 and 2; Track 2 and 3; Tracks 1, 2 and 3;

- ● setting software can be provided to set magnetic card recording format, character set, etc.

- ● power：DC+5.0V。

- E: iButton characteristics：

- ● read time within 5ms;

- ●Compliance with Dallas standards.

iButton card reader appearance and dimension diagram of structure

- F: Contact IC card reading and writing features:

- ● support CPU CARDS and multiple storage CARDS and logical encryption CARDS in accordance with iso7816-1/2/3 standards (which can be extended as required);

- ●TTL level UART communication interface;

- ● semi-settling IC card holder in Hamburg, Taiwan, life of card: ≥500000 times;

- ● good electromagnetic compatibility, anti-static design (refer to GB/T 17626 related standards);

- ● power supply: DC+5V ±10%.

- G: RFID card reader features:

- Standard, ISO/IEC14443A, ISO/IEC14443B, ISO/IEC15693;

- ● support card type: Mif are One S70; Mifare One UltraLight. Mifare One S50; Mifare Pro; AT88RF020; 66 cl160s; SR176; SRIX4K (TYPE B); I - Code2; TI RFid tag-it hf-i, EM4135,EM4034, etc. (other CARDS can be customized);

- ● communication interface: HID-USB;

- ● dimensions: 68*27*11mm;

- ● operating temperature: -20-70℃;

- ● working humidity: 10-90% no solidification.

H: Environment characteristics:

● working environment temperature: -20℃ - +50℃;

● storage temperature: -20℃ - +55℃;

● working humidity: 10%-90% no solidification;

● atmospheric pressure: 86-106kpa.

I: Application field

● access control attendance system

●POS terminals

● banking system

●ID authentication

● multimedia inquiry machine

J: online instructions

PC

USB port

Multi-function Reader

K: USB cable definition

| Pin | Signal |
| --- | --- |
| 1 | USB_VCC |
| 2 | USB_D- |
| 3 | USB_D+ |
| 5 | USB_GND |

L:list of readable character sets

6.1 Extension to ISO character set (ABA1 character set) :

| ASCII code | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | : | ; | < | = | > | ? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Record format | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | e | F |
| HEX code | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 3A | 3B | 3C | 3D | 3E | 3F |

6.2  Extensions to the IBM character set (ABA2 character set) :

| ASCII code | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | : | # | @ | ' | = | ? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Record format | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |
| HEX code | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 3A | 23 | 40 | 27 | 3D | 3F |

Note: for characters not in the above character set whose value is greater than 1FH, merge them to 30H; If it's less than 20H, ignore it.

M:software setting

- once the application software is installed, run the desktop generated DEMO. You will see the communication port selection screen. Note: the current device only supports usb-hid mode. < Picture 1 >

- < Picture 1 >

Illustration:

“①” ：Select the DEMO operation interface language.

“②” ：communication port selection.

“③” ：DEMO finds and connects MSR006 device.

“④” ：exit the DEMO.

- After connecting MSR006 successfully, enter the Settings page. The setup page consists of the "ISO" setup TAB, "JIS2" setup TAB, "iButton" setup TAB, "IC_CARD" function test TAB, and function buttons, as well as the main menu options for function buttons. Details will be given below.

- ⑴： “ISO” Setting TAB: < figure 2: >

< figure 2>

Illustration:

“①”：Corresponding to one, two, three track data read or not switch.

“②”：corresponding to the setting of SS< start symbol > of one, two and three magnetic CARDS respectively.

“③”：ES< end > setting corresponding to one, two and three magnetic CARDS respectively.

“④”：the setting of whether to connect to enter after track data of one, two and three respectively.

“⑤”：read the ISO format magnetic card, returned data header. < define method in appendix >

“⑥”：data separation between tracks returned when reading an ISO format magnetic card. < define method in appendix >

“⑦”：read the ISO format magnetic card, return the data tail. < define method in appendix >

“⑧”：“Enable Beep”corresponding to read card whether the buzzer sound.

“Support IBM Card” whether supports IBM format magnetic Card.

“Output Data For Any Valid Track”

“Reading Sequence” track Sequence selection.

“⑨”：language choice.

⑵： “JIS2” Setting TAB:< figure 3: >

< figure 3: >

Illustration:

“①”：The SS< start character > setting corresponding to the JIS2 magnetic card.

“②”：set ES< start symbol > corresponding to JIS2 magnetic card.

“③”：the data header returned when reading the magnetic card in JIS2 format. < define method in appendix >

“④”：reads the JIS2 format magnetic card, returns the data tail. < define method in appendix >

⑶： “iButton” Setting TAB:< figure 4: >

< figure 4>

Illustration:

“①”：The SS< start character > setting corresponding to iButton.

“②”：ES< start character > setting corresponding to iButton.

“③”：iButton's "Attach" and "Remove" data sending methods.

“④”：in the state of iButton’ s“Attach”, return back to the begin of data<define method in appendix >

“⑤”：in the state of iButton’ s“Attach”, return back to the end of data<define method in appendix >

“⑥”：in the state of iButton’ s“Attach”:

“Extract Form” Start offset.

“Length” Length of data returned

“Enable Beep” Whether the buzzer sounds or not.

“⑦”：in the state of iButton’ s“remove”, return back to the begin of data<define method in appendix >

“⑧”：in the state of iButton’ s“remove”, return back to the end of data<define method in appendix >

“⑨”：in the state of iButton’ s“remove”：

“Extract Form” Start offset.

“Length” Length of data returned

“Enable Beep” Whether the buzzer sounds or not.

⑷： ”IC_CARD” Functional test TAB：< figure 5:>

< figure 5>

Illustration:

“①”：IC card power on function test.

“②”：IC card power down function test.

“③”：IC card APDU function test.

“④”：specific communication data in IC card test process.

⑸： Function buttons and the main menu of function buttons:< figure 6:>

< figure 6>

Illustration:

“①”：“Load”：from MSR006 set to DEMO.

“②”：“Open File”：from file read the setting toDEMO.

“③”：“Save File”：save the setting of DEMO to file.

“④”：“Normal Mode”：let MSR006 go into the testing mode< only valid for ISO format >

“⑤”：“Test Mode”：let MSR006 go into the testing mode< only valid for ISO format >

“⑥”：“Save”：save DEMO the setting to MSR006.

“⑦”：“Set Default”：Set MSR006 as the default parameter.

“⑧”：exit to DEMO。

⑹：helpful functional menu：< figure 7:>

< figure 7>

Illustration:

“①”：“Version” menu，receive MSR006 model no..

“②”：“About”menu，receive DEMO relevant information.

Appendix A:

ASCII defines methods

When the user defines "Header", "Separator", "Suffix", and "Prefix", the ASCII mode is selected to enter the following screen. < figure 8>

< figure 8>

Illustration:

(1) : the user directly input characters in the edit box, and then click "OK" button, you can successfully define.

(2) : special function character definition.

| Special character | Definition |
| --- | --- |
| Enter | \n or \N |
| Esc | \e or \E |
| Tab | \t or \T |
| \ | \\ |

(3): The current version of ASCII string definition, the maximum length is 20.

Appendix B:

The definition of CODE method

When the user defines "Header", "Separator", "Suffix" and "Prefix", the CODE mode is selected to enter the following interface.< figure 9>

< figure 9>

Illustration:

(1) : double-click the CODE item on the right, DEMO will fill in the editing box on the left with the double-click content of the user, and then click "OK" button, you can successfully define.

(2): the current version of the CODE definition, the maximum length is 20.